电子系统设计基础

组员：杜斯博 信息2301 2234112313

敖凯航 信息2301 2234214025

刘轩熙 信息2303 2224312764

- 实验目的

- 学习并了解51单片机的基本结构和引脚相关知识。

- 学习并了解51单片机系统板相关知识。

- 学习51单片机程序设计开发流程，锻炼使用51单片机设计程序实现功能的能力。

- 实验任务

实验名称：流水灯。

使用51单片机实现下列功能：

- 8个7段数码管以流水灯的形式显示当前按键值。

- 8个7段数码管呈现同时显示当前按键值的效果。

- 8个7段数码管滚动显示自己学号后8位。

- 要求不同功能在一个main函数中实现，使用按键切换功能。

- 实验原理

工具：    Keil-C51版本（51单片机的IDE开发环境，ch340驱动

参考文件：keil-C51的安装过程.pdf 、keil-C51新建工程.pdf

语言：    C语言、汇编语言或两者混合

开发流程：新建工程、添加设计文件、编译、调试下载

在设计过程中应注意：

- 程序的入口为main函数，main函数中以while死循环结尾；

- 键值获取为4*4键盘地址：0x8000，并注意消抖。（键值获取程序在pdf中已经给出）

- 七段数码管的显示驱动，应实现扫描显示效果和同时显示效果。（单管显示控制程序在pdf中已经给出）

- 实验结果

基本实现了三项任务要求。

- 在程序下载烧录后，单片机数码管被点亮并实现第一功能，以流水灯形式显示检测到的按键值。此后再按下按钮10进入第一功能。

- 按下按钮11进入第二功能，八个七段数码管同时呈现同时显示当前按键值。

- 按下按钮12进入第三功能，八个七段数码管滚动显示学号后八位，实验中以组员“34214025“为例。

- 实验感悟

实验旨在学习应用单片机结构、系统板及程序开发流程，基于51单片机文具盒系统实现了三个流水灯功能：流水灯显示按键值、八位数码管同步显示按键值及滚动显示学号后八位。使用Keil-C51开发环境，结合C语言编写程序，通过按键消抖和动态扫描驱动数码管显示。通过实践，加深了我们对单片机系统开发流程的理解，提升了软硬件协同设计能力和调试程序的能力，为后续复杂系统开发奠定了基础。

个人感悟：

实验考验对单片机程序设计流程的熟悉程度，在课下我们自主学习了很多有关单片机编程的相关知识，才最终能完成这次实验。在实验过程中我们遇到了很多问题，比如下载烧录后单片机数码管不亮、按键失效、并通过第二个功能检验出了一部分按键存在失灵和接触不良的险象。实验过程中老师的指导和pdf里的代码给予了我们很大帮助，通过不断调试和更改delay延迟，最终实现了以上的任务要求，在实验完成时很有成就感。

附注：实验代码：

main.c:

#include "seg.h"

#include "keycode.h"

#include <MATH.H>

static unsigned char stu[8] = {5, 2, 0, 4, 1, 2, 4, 3}; //学号后八位，倒置

void main(void){

unsigned char state=10;

unsigned char key_code;

unsigned char key_real;

unsigned char k = 0;

unsigned char flag = 0;

unsigned char i = 0;

unsigned int j = 0;

unsigned int m = 0;

while(1){

switch(state){

case 10:

i=0,k=0,j=0;flag=0;

while(1){

key_code = GetKeyCode();	//键值需匹配数字

if (key_code != 0x00){		//按键按下

flag = 1;

key_real = key_code;

break;

}else{						//按键未按下

flag = 0;

break;

}

}

k = GetNum(key_real);

if(GetNum(key_real)==10||GetNum(key_real)==11||GetNum(key_real)==12){

state=k;

break;

}

if (flag == 1){				//按键按下

for (i = 0; i < 8; ++i){

for (j = 0; j < 500; ++j){

Seg7Show(led_table_loc[i-1],k);

Seg7Show(led_table_loc[i],k);

Seg7Show(led_table_loc[i+1],k);

}

}

for (i = 8; i > 0; --i){

for (j = 0; j < 500; ++j){

Seg7Show(led_table_loc[i+1],k);

Seg7Show(led_table_loc[i],k);

Seg7Show(led_table_loc[i-1],k);

}

}

XBYTE[0x9000] = 0x00;

}

break;

case 11:

i=0,k=0,j=0;flag=0;

while(1){

key_code = GetKeyCode();	//键值需匹配数字

if (key_code != 0x00){		//按键按下

flag = 1;

key_real = key_code;

break;

}else{						//按键未按下

flag = 0;

break;

}

}

k = GetNum(key_real);

if(GetNum(key_real)==10||GetNum(key_real)==11||GetNum(key_real)==12){

state=k;

break;

}

// Delay(10);

if (flag == 1){				//按键未按下

for (i = 0; i < 8; ++i){

for (j = 0; j < 50; ++j)

Seg7Show(led_table_loc[i],k);

}

}

break;

case 12:

while(1){

i=0,k=0,j=0;flag=0;

for (j = 0; j < 8; ++j){

for (k = 0; k < 60; ++k){

for (i = 0; i < 8; ++i){

Seg7Show(led_table_loc[i],stu[(i - j + 8)%8]);

for(m=0;m<200;m++);

}

}

}

}

break;

}

}

}

seg.h:

#ifndef SEG_H_

#define SEG_H_

#include <REG51.H>

#include <ABSACC.H>

static unsigned char led_table_num[17] = {0x3f, 0x06, 0x5b, 0x4f,

0x66, 0x6d, 0x7d, 0x07,

0x7f, 0x6f, 0x77, 0x7c,

0x39, 0x5e, 0x79, 0x71, 0x00};

static unsigned char led_table_loc[8] = {0x01, 0x02, 0x04, 0x08,

0x10, 0x20, 0x40, 0x80};

void Delay(unsigned int t);

void Seg7Show(unsigned char which_seg, unsigned char which_num);

void Seg7Close(unsigned char which_seg, unsigned char which_num);

#endif

seg.c:

#include "seg.h"

void Delay(unsigned int t){     // 晶振频率11.0592MHz，一个机器周期1.085us，922*t=922时，延时1ms

unsigned int i;

for (i = 0; i < 922*t; i++)

{

;

}

}

void Seg7Show(unsigned char which_seg, unsigned char which_num){

XBYTE[0x8000] = which_seg;

XBYTE[0x9000] = 0x00;

XBYTE[0x9000] = led_table_num[which_num];

}

void Seg7Close(unsigned char which_seg, unsigned char which_num){

XBYTE[0x8000] = which_seg;

XBYTE[0x9000] = 0x00;

}

keycode.h:

#ifndef KEYCODE_H_

#define KEYCODE_H_

#include <REG51.H>

#include <ABSACC.H>

unsigned char GetKeyCode(void);

unsigned char GetNum(key);

#endif

keycode.c:

#include "keycode.h"

#include "seg.h"

unsigned char GetKeyCode(void){

unsigned char line = 0x00;

unsigned char col = 0x00;

unsigned char scan_code = 0x01;

unsigned char key_code;

XBYTE[0x8000] = 0xff;

col = XBYTE[0x8000] & 0x0f;

if (col == 0x00){

key_code = 0x00;

}else{

while ((scan_code & 0x0f) != 0){

line = scan_code;

XBYTE[0x8000] = scan_code;

if((XBYTE[0x8000] & 0x0f) == col){

break;

}

scan_code = scan_code << 1;

}

col = col << 4;

key_code = col | line;

}

return key_code;

}

unsigned char GetNum(key){

unsigned char k;

switch (key)

{

case 0x11:

k = 0;

break;

case 0x21:

k = 1;

break;

case 0x41:

k = 2;

break;

case 0x81:

k = 3;

break;

case 0x12:

k = 4;

break;

case 0x22:

k = 5;

break;

case 0x42:

k = 6;

break;

case 0x82:

k = 7;

break;

case 0x14:

k = 8;

break;

case 0x24:

k = 9;

break;

case 0x44:

k = 10;

break;

case 0x84:

k = 11;

break;

case 0x18:

k = 12;

break;

case 0x28:

k = 13;

break;

case 0x48:

k = 14;

break;

case 0x88:

k = 15;

break;

default:

k = 0;

break;

}

return k;

}